# Introduction

## Colorectal Cancer in the United States

Colorectal cancer (CRC) is one of the predominant forms of cancer within the United States, being the 3rd most common cause of cancer and cancer death in both men and women(Siegel et al., 2019).  While 25% of patients present with colorectal liver metastasis (CRL-mets) at the time of diagnosis, up to 60% will develop CRL-mets over the course of their disease(Alper Sag et al., 2016).

## Detection of Colorectal Liver Metastasis (CRL-mets)

While several treatment options are available for CRL-mets, they all first rely on proper identification of the disease. The gold standard of identifying CRL-mets consists of a contrast-enhanced CT (CECT) scan[ref]. A CECT often consists of three phases: an arterial phase where the contrast is most prominent in the descending aorta, the portal-venous phase, where contrast is mostly present within the portal-vein, and a washout phase. CRL-mets often appear as hypo-dense regions in the arterial and portal-venous phase of the image, in stark contrast to the normal liver tissue, Figure xxx.

Figure 1: Portal venous phase of contrast enhanced CT scan. Disease contoured in red is easily distinguishable from normal liver.

Challenges with detecting CRL-mets are mostly with respect to the diseases relatively small size and large search area in the liver. Furthermore, interventions such as radiation therapy, radiofrequency ablation, and microwave ablation, require accurate segmentations of the disease site for dose evaluation or image guidance. To date, automated liver disease segmentation tasks have largely focused on primary liver disease [ref], although new competitions such as the Liver Tumor segmentation (LiTs) challenge have contained data of both primary and secondary liver cancers.

## Ablation Therapies

Specifically for ablation interventions, segmentations of the disease in the pre-treatment imaging, and the ablation cavity in the post-treatment imaging, are important for evaluating the minimum delivered ablation margin, Figure xxx. Both radiofrequency and microwave ablation interventions require that a minimum margin be delivered around the gross disease to ensure that all microscopic disease is treated. While automated workflows exist to map the disease from the pre-treatment to the post-treatment imaging to assess treatment efficacy, these still require manual segmentations of both disease and ablation zone.

Figure 2: Late portal venous phase of contrast enhanced CT scan. Ablation cavity contoured in blue is easily distinguishable from normal liver.

We hypothesize that fully convolutional neural networks, trained in a unique fashion to account for the challenges of disease size with respect to the overall liver, can provide rapid and accurate segmentations of both CRL-mets and ablation generated cavities. The creation of which would enable dose escalation studies of CRL-mets in radiation therapy, and facilitate margin evaluations in ablation therapy.

# Materials/Methods

## Data

### Quantitative Training/Validation/Test

To ensure that our model creatoin is reproducible at other institutions, our training\validation and quantitative test data was collected from the publically available Liver Tumor Segmentation (LiTS) challenge (https://competitions.codalab.org/competitions/17094). This data consisted of 130 CECT scans with liver tumors of primary liver cancer and secondary liver metastasis. Ground truth segmentations of the liver and disease were provided by the challenge. Each image was evaluated by xxx, and removed if the disease was hyper-enhancing or not secondary metastasis, or if the image acquisition parameters (slice thickness, pixel spacing) were not correctly included. A total of 93 patients remained, which were then distributed into 57 patients for training, 10 patients for validation, and 26 patients for a completely withheld test set. The image acquisition parameters for the training, validation, and test set can be seen in Table xxx.

Table 1: Image acquisition parameters from LiTs Training, Validation, and Test set.

| Folder | Slice Thickness | Pixel Size X | Pixel Size Y |
| --- | --- | --- | --- |
|  | Mean (Min, Max) |  |  |
| Training | 1.635 (0.7, 5.0) | 0.75 (0.597, 0.976) | 0.75 (0.597, 0.976) |
| Validation | 2.017 (0.7, 5.0) | 0.78 (0.675, 0.976) | 0.78 (0.675, 0.976) |
| Test | 1.41 (0.7, 5.0) | 0.76 (0.644, 0.968) | 0.76 (0.644, 0.968) |

#### Quantitative Evaluation

Best model predictions on the test set are assessed based on the Dice Similarity Coefficient (DSC), Mean Surface Distance (MSD), and Hausdroff Distance. To remove bias of patients with multiple small disease sites, evaluations were performed on both a disease site-by-site basis, and with comparison of patient-by-patient.

For the site-by-site comparison, each non-connected disease site in the ground-truth is considered as an independent test case. Ground-truth and prediction are evaluated by removing all but the closest (determined by difference between centroid of ground-truth and prediction regions) prediction region.

For patient-by-patient comparison, metrics are computed by comparison of the ground truth and the prediction on the entire liver.

### Qualitative Test Data

Quantitative metrics are not always sufficient to assess a segmentation algorithms efficacy. For large structures, a high Dice Similarity Coefficient (DSC) can hide clinically impactful inaccuracies, while for a smaller structures a low DSC could be overly critical. Therefore, we wanted to acquired a qualitative assessment of both CRL-met segmentation and ablation segmentation by two trained interventional radiologists.

#### Qualitative Evaluation

We acquired 18 patients who had previously undergone thermal ablation therapy at our institution. These patients each had two CECT scans, one pre-ablation to identify the disease, and one post-treatment to identify the ablation zone. Two interventional radiologists independently reviewed the program’s segmentation of the disease and ablation zone on pre-treatment and post-treatment scans, scoring each contour on a scale of 1-5. The criteria for the scale can be seen in Table xxx.

Table 2: Likert scoring for assessment of colorectal liver metastasis (CRL-met) and ablation contours

| Score |  | Criteria |
| --- | --- | --- |
| 5 | Excellent | Negligible changes, could be used clinically without edits |
| 4 | Good | Minor changes on < 4 slices, or that would take less than 10 seconds to fix |
| 3 | Fair | Edits needed on multiple slices, but less than 50% |
| 2 | Poor | Greater than 50% of slices need editing |
| 1 | Very poor | Contours are completely useless, missed disease |

Two patients had two disease sites present, and so a scoring for each disease site was performed. Scans were removed if the radiologists felt sufficient contrast was not present.

## Data Pre-Processing

### Intensity Manipulation

Initial attempts to normalize the data based on a global mean and standard deviation resulted in extreme outliers when the contrast within the liver was high or low, Figure xxx. To best separate the normal tissue from the disease, we acquired a patient specific mean and standard deviation based on the FWHM of values within the contoured liver. Visually, this appeared to result in a robust method where the disease appeared consistently darker than the normal tissue. Image intensity outside of the liver is set to be equal to 0 after normalization.

Figure 3: Images normalized based on global mean and standard deviation of liver tissue. Left, patient with mean liver HUs near the global mean. Right, patient with mean liver HU < 3 sigma from global mean.

### Voxel Size Resampling

A known weakness of convolutional neural networks is instability with varying image sizes. Our first naïve approach to this task did not involve resampling the images in any form, and the results showed a clear improvement in predictions when the images has slice-thicknesses similar to the mean of the training dataset. In our final iteration, we re-sampled all training and validation images to be 1mm in slice thickness using bi-linear interpolation. Likewise, we bi-linearly interpolate the ground-truth segmentations of the disease, providing the model with values ranging from 0-1 for each part of the disease, Figure xxx. This bi-linear interpolation of ground truth was performed after our first naïve approach of nearest neighbor showed routine under-segmentation of disease.

Figure 4: Linearly resampled ground-truth overlaid onto CT. Green region represents 60% certainty, yellow is 80%, and red is 100%.

For all test assessment, predictions were resampled to the original slice thickness and compared against the unchanged ground truth.

### Training Image ‘Cubes’

Relative sizes of annotations is a well-known concern within image segmentation. Smaller structures are inherently worth less than large structures without a class weighting and a categorical cross-entropy loss. Unfortunately, simple class weighting would invariably weigh the more heavily diseased patients over less diseased patients, Figure xxx.

Figure 5: Representation of disease size issue. If class weighting is used to increase the value of ‘disease’, the image on the left will be worth more in the training process than the one on the right.

To this end, we break up each independent disease site within a patient into individual ‘cubes’ of 32x120x120 voxels. Note that this means disease not centered on the cube is still properly defined as disease, Figure xxx. After extraction, the training dataset consists of 572 unique image cubes.

Figure 6: Liver distributed into individual cubes of 32x120x120. Disease is labeled as disease regardless of center of cube.

## Architectures

We initially attempted to train a 2D model, the DeepLabV3+ with the Xception backbone; however, our inability to converge and over-abundance of false-positives led us to pursue 3D approaches.

We investigated three broad architectural styles with several fine-tuning variations. The basic framework for our architecture is the form of a UNet, defined as a number of convolution blocks per layer, number of layers, convolution block lambda (which is the increase in number of convolution blocks per layer), maximum number of convolution blocks, and presence of atrous convolutions, Figure xxx.

Figure 7: Basic framework of architecture. This would be defined as a three layer, two convolution block (large green arrow), convolution block lambda equal to one (increase in number of convolution blocks each layer), and max convolution blocks equal to three.

A comparison of all three approaches in how the convolution blocks (green bounded region in Figure xxx) were created is shown in Figure xxx.

### Vanilla UNet

The first style we investigated is a ‘vanilla’ UNet, where concatenations or additions were used to combine equal size feature maps from the encoder to the decoder side.

Vanilla Architecture Parameters:

- Number of layers were varied from 1, 2, 3, and 4

- Initial number of convolution blocks were varied from 1, 2, and 3

- The increase in number of convolution blocks (convolution lambda) for each layer was varied from 0, 1, and 2

- Maximum number of convolution blocks was 4

- The number of initial filters was varied from 8, 16, and 32

- The maximum number of filters was varied from 32, 64, and 128

### Residual UNet

Next, we investigated using residual connections for each convolution block in each layer, with motivation from the ‘ResNet’ architecture(He et al., n.d.). Residual connections have the benefit of allowing a ‘flow’ of loss from previous convolutions. This allows the model to create skip-connections over convolutions that might not be necessary. Unfortunately, convolutions after the skip-connection are a combination of both the skip connection and direct line.

Residual Network Architecture Parameters:

- Number of layers were varied from 1, 2, 3, and 4

- Initial number of convolution blocks were varied from 1, 2, 3, and 4

- The increase in number of convolution blocks (convolution lambda) for each layer was varied from 0, 1, and 2

- Maximum number of convolution blocks was 4

- The number of initial filters was 32

- The maximum number of filters was 128

### Dense UNet

A more complete ‘flow’ of loss from previous convolutions can be realized with the DenseNet architecture(Huang et al., n.d.). This architecture style allows previous convolutions to be re-used, removing the potential necessity of the model to re-create feature maps and enabling more diversified feature creation. Another benefit of this method is that the number of filters can be significantly reduced since previous convolutions are re-used; only increasing the number of filters by a small amount each time (referred to here as ‘growth rate’).

Dense Network Architecture Parameters:

- Number of layers were varied from 2, and 3

- Initial number of convolution blocks were 2

- The increase in number of convolution blocks (convolution lambda) for each layer was varied from 0, and 1

- Maximum number of convolution blocks was 4

- The number of initial filters was 8, 12, 16, and 32

- Filter growth rate was 0, and 4

- The maximum number of filters was infinity

A representation of the differences in how convolutions and connections occur from the ‘vanilla UNet’, ResNet UNet, and Dense UNet can be seen in Figure xxx.

Figure 8: Difference in convolution block. Left, ‘Vanilla’, previous feature maps are convolved and activated. Middle, ‘Residual’, previous feature maps are directly added to convolutional output in a ‘skip’ connection before activation. Right, ‘Dense’, previous feature maps are continually concatenated together before activation and convolution.

## Training

All model training was performed using NVIDIA-Tesla V100 32GB GPUs(NVIDIA Tesla V100 | NVIDIA, n.d.). All model creation, training, optimization, and evaluation was performed using Tensorflow version 2.2.0 (Abadi et al., n.d.). Models were optimized using a sparse categorical cross entropy loss and Adam optimizer. Mixed precision was enabled to speed up the training process.

The model takes two inputs, an image and a binary mask of the liver. The liver mask automatically assigns background to any voxel not present within the liver, as any disease should be within the provided liver contour. Training involved passing batches of 8 or 16 ‘cubes’ of 32x120x120x1 images and liver masks to the model.

### Hyper-Parameters

The learning rate is known to be one of the most important variables in determining a model’s ability to converge, and optimal rates will vary based on the architecture depth (layers), number of convolution blocks, and number of filters in each block. For each model, a variation on the cyclical learning rate(Smith, n.d.) is used (link to GitHub, anonymized for now), where the model starts training at a minimum learning rate before linearly increasing to the maximum and back down to the minimum, Figure xxx. Our version allows the user to select the fraction of previous max (shown as ½), and allows the user to define a fraction that the minimum learning rate decreases by each cycle.

Figure 9: Cyclical Learning Rate for 1e-10 to 1e-9, assuming a step size of 5 and step size factor of 5. (Top)  Linear representation and (Bottom) log representation.

The minimum and maximum learning rate are found via small adaptations of code presented here https://www.pyimagesearch.com/2019/08/05/keras-learning-rate-finder/, our own variation of the code can be found at our github page, link to github anonymized*. This code functions by slowly increasing the learning rate from a very low value (~1e-10) to a large value (~1-e-1), and training solely on the training dataset. The point where the loss begins to decrease refers to the minimum learning rate, and the point where it begins to increase again is the maximum learning rate, Figure xxx.

Figure 10: Learning Rate Finder figure. Learning rate is gradually increased from 10e-7 to 10e-0, the point where loss begins to decrease is the minimum learning rate (Min LR), the point where it begins to increase is the maximum learning rate (Max LR).

A unique minimum and maximum learning rate were found for each architecture parameter, for reach style of architecture.

### Optimization

Each unique model was run three independent times with randomly initialized variables for 101 epochs. Plotnine and Tensorflow’s Tensorboard with hyperparameter tuning were used to identify any potential trends and direct model training. The final model was selected based on the Dice Similarity Coefficient (DSC) between the validation set prediction and ground truth during the training process.

### Visualization of Training

As disease sites are quite small and we were concerned about metrics being misleading to actual segmentation accuracy, we implemented a method of creating prediction images on the validation data during training, Figure xxx. This method can be easily implemented as a Tensorflow callback, *link to github, left out for anonymization*.

Figure 11: Example of visualization of prediction on validation data during training. Top row: Image being fed into model, masked to 0 outside of the liver. Middle row: Ground truth of disease contours, resampling to 1mm slicing will also interpolate ground truth, right image shows some sampling. Bottom row: Prediction of disease contours, set with threshold of 0.5 to binary mask.

## Model Evaluation

### Determining Seed and Threshold Values

Predictions of disease from the model come in a range of 0-1. Binary masks are generated via seed-point identification and growth of the output prediction, Figure xxx.

The ideal seed value and threshold values were selected based on the highest DSC in the validation dataset. Seed values ranged from 0.8-0.99 in 0.01 increments, and threshold ranges from 0.1 to 0.8 in 0.05 increments, a graphical representation of DSC for seed-value and threshold range can be seen in Figure xxx, where green corresponds to higher DSC.

# Results

## Quantitative

The best model, with a DSC of 0.79 on the validation dataset during training, was found to be DenseNet model, with two layers, two convolution blocks, convolution lambda equal to one, and eight filters, Figure xxx. The model consists of xxx parameters. From this model, a seed-value of xxx and threshold value of xxx resulted in the highest overall DSC in the validation dataset.

### Disease Site-by-Site evaluation

The average (min, max) DSC and MSD on a site-by-site basis for the completely withheld test set (n=xxx lesions) was xxx and xxx, respectively. The DSC for each disease site and volume of the disease site can be seen in Figure xxx.

### Disease Patient-by-Patient evaluation

The average (min, max) DSC and MSD on a patient-by-patient basis was xxx and xxx, respectively. Note that a single patient might have multiple disease sites of varying size. The DSC for each patient can be seen in Figure xxx.

## Qualitative

Using the Likert scoring system shown in Table xxx, the average (min, max) Likert score for all qualitative patients gross tumor volume (n=19) for each radiologist and their combined score was Radiologist One: 4.63 (3, 5), Radiologist Two: 4.80 (4, 5), Combined: 4.71 (3, 5). The average (min, max) Likert for each qualitative patient ablation volume (n=12) for each radiologist and their combined was Radiologist One: 4.00 (1, 5), Radiologist Two: 3.77 (1, 5), Combined: 3.88 (1,5), Table xxx.

Table 3: Likert score for Gross Tumor Volume and Ablation Zone segmentation for each Radiologist and their combined scores.

| Site (n) | Likert Score Average (Min, Max) |  |  |
| --- | --- | --- | --- |
|  | Radiologist 1 | Radiologist 2 | Combined |
| Gross Tumor Volume (n=19) | 4.63 (3, 5) | 4.80 (4, 5) | 4.71 (3, 5) |
| Ablation Zone (n=12) | 4 (1, 5) | 3.77 (1, 5) | 3.88 (1,5) |

A more detailed graph for each radiologist score on each gross tumor volume and ablation zone are shown in Figure xxx and Figure xxx, respectively.

Figure 12:Likert Score for each radiologist on each disease site. A higher value is associated with a better contour.

Figure 13: Likert Score for each radiologist on each ablation site. A higher value is associated with a better contour

Figure 14: DenseNet final architecture. Two layers, two convolution blocks in Layer 0, three convolution blocks in Layer 1, and eight initial filters.

# Discussion