# Abstract

Objectives: Liver ablation is an effective therapy for the treatment of colorectal liver metastasis. Identification of tumor and ablation zone boundaries are a critical and time-demanding step for assessing liver ablation efficacy. We hypothesize FC neural networks, trained using novel methods, will provide rapid and accurate identification and segmentation of CRLM and ablation-zones.

Methods: 92 patients from the Liver Tumor Segmentation(LiTS) challenge were acquired for training. For the evaluation, we acquired 15 patients from the 3D-IRCADb database, 18 patients from our institution (CRLM=24, ablation-zone=19), and submitted to the LiTS challenge(n=70). Four FC model styles were investigated: Standard-, Residual-, Dense-3D-UNet, and Hybrid-WNet. Qualitative evaluations of our institutional data were performed by two board certified radiologists(interventional, diagnostic), and a radiology-trained physician fellow, using a Likert scale 1-5.

Results: The most accurate model was the Hybrid-WNet. On a patient-by-patient basis in the 3D-IRCADb dataset, the median(min-max) Dice similarity coefficient(DSC) was 0.73(0.41-0.88), median surface distance was 1.75mm(0.57–7.63mm), and number of false-positive tumors identified  was 1(0-4) per case. In the LiTS challenge(n=70), the global DSC was 0.810.  The model sensitivity was 98%(47/48) for tumor sites ≥15mm in diameter. Qualitatively, 100%(24/24) of the CRLM and 84%(16/19) of the ablation-zones had Likert scores ≥4.

Conclusion: The Hybrid-WNet model provided fast(<30 seconds) and accurate segmentations of CRLM and ablation-zones on contrast-enhanced CT scans, with high concordance with physician reviews. This model might allow rapid tumor and ablation zone segmentations that might assist the interventional radiologist to evaluate ablation technical success intra-procedurally in an accurate and timely fashion.

Keywords: Radio-Frequency Ablation; Deep learning; Liver Cancer

Key points:

- A uniquely trained segmentation model was created using publically available data.

- The model predictions of colorectal liver metastasis and ablation zones were created in less than 30 seconds.

- Qualitative evaluation via interventional radiologist, diagnostic radiologist, and radiology trained physician fellow found 100% of the disease sites had a Likert Score greater than or equal to 4 out of a possible 5.

Abbreviations

PTA: Percutaneous ablation therapy

CRC: Colorectal cancer

CRLM: Colorectal liver metastasis

LRT: Loco-regional therapies

CECT: Contrast enhanced computed tomography

LiTS: Liver Tumor Segmentation

DSC: Dice similarity coefficient

# Introduction

Liver ablation is an effective and widely utilized curative-intent local therapy1–4 utilized on patients presenting with colorectal liver metastasis (CRLM). Among all the known factors associated with local tumor progression, minimal ablation margins is the only one that can be effectively modified by the interventional radiologist during liver ablation treatments2,5–8. In order to properly assess minimal ablation margins, segmentation of tumor on pre-ablation images as well of the ablation zone on post-treatment images are required9. This laborious process requires review of several imaging slices, is prone to inter-observer variability, and takes precious time away from the procedure. Moreover, similar to the CRLM, the ablation zone is hypo-enhanced on CECT images, making both indistinguishable. A clinical trial is underway (Identifier: NCT04083378)  to map the CRLM from pre-treatment to post-treatment imaging and assess treatment efficacy, but manual segmentations of both the disease and the ablation zone is still required10, adding time to the procedure.

To date, automated liver disease segmentation tasks have either largely focused on primary liver disease, or have not included qualitative evaluation of generated contours11–14. New institution- and society-sponsored competitions, such as the Liver Tumor segmentation (LiTS) challenge15 and the 3D-IRCADb0111 dataset have included data from both primary and secondary liver cancers, enabling investigation, development and comparison of automatic segmentation algorithms using public data. We hypothesized that fully convolutional neural networks, trained using novel methods to account for the challenges of varying disease size, will provide rapid and accurate identification and segmentation of both CRLM and ablation zones. We believe that this approach will facilitate the automated detection of CRLM, radiation treatment planning for CRLM, and the evaluations of margin in ablation therapy.

# Materials and Methods

## Quantitative Training, Validation, and Testing

To ensure reproducibility by other institutions, we used data provided by the publicly available LiTS challenge15 for training, which was reviewed by XXX. LiTS consists of CECT scans from 131 patients with primary and secondary liver disease. Ground truth segmentations of the liver and disease were provided. Hyper-enhancing disease, inconsistent with the intent of developing a CRLM model for hypo-enhancing metastases, was removed. In addition, 4 cases were excluded due to lack of image acquisition parameters necessary for reconstructing a geometrically accurate depiction of the patient. 92 patients remained.

For withheld validation, we acquired the 3D-IRCADb01 publically available dataset16. Five of the patients from the 3D-IRCADb01 dataset were excluded: patients 5, 11, and 20 had no CRLM, patient 12 had a large cystic lesion at the base of the liver, and patient 18 had CRLM with atypical enhancement pattern. Table 1 shows the image acquisition parameters for the training, validation, and test sets.

We leveraged the hypo-intense similarities in appearance between CRLM and ablation zones to employ transfer learning to apply the trained model for the task of ablation zone segmentation.

## Data Pre-Processing

### Image Intensity Manipulation

We calculated a patient-specific mean and standard deviation for normalization on the basis of the Full-Width-Half-Max of the values within the liver, this reduced outliers as compared to using a global mean and standard deviation. The image intensity outside of the masked liver was set to be equal to 0.

### Voxel Size Resampling

We resampled all training and validation images and ground-truth segmentation to 1mm image thickness using bi-linear interpolation.

### Training Image “Slabs”

To account for disparities in class representation, where smaller structures are inherently “worth less” than large structures, we distributed our training into unique “slabs”. Simple class weighting would not solve this problem as it would result in the model weighing patients with larger tumor burden as more important. Each independent disease site was divided into “slabs” of 32x120x120 voxels. Figure 1 illustrates several disease slabs for one patient. After extraction, the training dataset consisted of 572 unique samples. The validation and test set were not broken into slabs, with the entire liver being passed at once for evaluation and testing.

## Architectures

Four architectures were investigated: Standard 3D-UNet, Residual 3D-UNet, Dense 3D-UNet, and Hybrid-WNet (pre-trained Standard 2D-UNet with a 3D-DenseUNet). The basic framework remained the same for the Standard, Residual, and Dense 3D-UNets (Figure 2A). The differences in the Standard, Residual, and Dense 3D-UNets are represented in Figure 2B. The Hybrid-WNet architecture is shown in Figure 3. A list of parameters for each architecture is listed in Table 2.

### Residual 3D-UNet

The Standard 3D-UNet was expanded to include residual connections for each convolution block in a layer, with motivation from the ‘ResNet’ architecture17. Residual connections have the benefit of allowing a “flow” of loss from previous convolutions. This allows the model to create skip-connections over convolutions that might not be necessary.

### Dense 3D-UNet

A more complete “flow” of loss from previous convolutions can be realized with the DenseNet architecture18. This architectural style allows previous convolutions to be re-used. The reuse of previous convolutions allows the number of filters to be significantly reduced, the increase in total number of filters is referred to here as the “growth rate”.

### Hybrid-WNet

The architecture combines 2D features extracted from the pre-trained 2D DenseNet-12118 in Tensorflow19 with a 3D convolutional neural network. We coined the term Hybrid-WNet on the basis of the W-shaped appearance of two UNets beside each other (Figure 3).

Our novel Hybrid-WNet architectural style was inspired by Li et al.13 with significant alterations. First, in architecture training, we broke the training process into four steps: 1) training only the new decoding side of the DenseNet 121, 2) training the entire DenseNet121, 3) training only the 3D network with the extracted 2D features, and 4) entire end-to-end training. By breaking up the training process in this fashion, we ensured that we could use high learning rates without the risk of “untraining” pretrained layers. Second, our 3D DenseNet contained truly dense layers, with extracted features shared throughout the entirety of each layer; this enabled the use of significantly fewer features.

## Model Training

All model training was performed using NVIDIA-Tesla V100 32GB GPUs20. All model creation, training, optimization, and evaluation was performed using Tensorflow2.2.021. Models were optimized using a sparse categorical cross entropy loss and Adam optimizer. Mixed precision was enabled to reduce the training time.

The model was trained with two inputs: CT image and binary mask of the liver. The mask automatically assigns a background to any voxel outside the liver. Training involved passing B*N*H*W*C tensors to the model, where the (B)atch varied from 8, to 16, the (N)umber of slices was 32, the (H)eight was 120, the (W)idth was 120, and the (C)hannels was 2 (image and liver mask). Thus, a single pass might be 8x32x120x120x2 in size.

### Training the DenseNet121 UNet

When training the 2D aspect of the Hybrid-WNet, 3D slabs were reshaped into stacked 2D images. For example, a batch of 8x32x120x120x1 would be transformed into 8*32x120x120x1. In the first training iteration, all weights on the pre-trained encoding architecture are frozen. Next, all weights were made trainable, allowing the model to tweak any pre-trained layers.

### Training the Combined 2D-3D WNet

After training the 2D part of the W-Net, we concatenated the 2D features to the input of the 3D model. All weights from the 2D network were initially frozen, and only the 3D model trained. Next, all weights were unfrozen, allowing the model to be fine-tuned.

## Hyper-Parameters

For training each model, a variation in the cyclical learning rate22 was used (GitHub link: anonymized for review), with linear increase and decrease between min and max. Optimal learning rates vary based on each architecture parameter, with the minimum and maximum learning rates identified using an in-house function (Githb link: anonymized for review), Supplementary Figure 1.

## Model Optimization

We ran each model three times using randomly initialized variables to reduce likelihood of poor initialization. Plotnine and Tensorflow’s Tensorboard (https://github.com/tensorflow/tensorboard) was used to identify trends and direct model training. The final model was selected on the basis of the Dice similarity coefficient (DSC) between the validation set and the ground truth.

Prediction images were visualized during training to assess the training process (Figure 4) as a Tensorflow callback (Githb link: anonymized for review).

## Model Evaluation

### Seed and Threshold Values

The disease predictions from our model ranged from 0-1. Unfortunately, the most inferior and superior aspects of a disease site often had a lower probability than the center of the disease. For this reason, we investigated seed-point growth for the final prediction. Seed points were created to define the likely starting point of a disease site, and then grown outwards to a threshold value. We investigated seed values from 0.25-0.95 in 0.01 increments, and threshold values from 0.2 to 0.8 in 0.01 increments.

## Quantitative Evaluation

Model performance was evaluated on the test set using the DSC and Median Surface Distance between the manual and predicted segmentations. We report on predictions in a disease site-by-site, patient-by-patient, and “global” basis.

For the site-by-site comparison, each non-connected disease segmentation of the test patients was considered an independent case. We computed the metrics between the manual segmentation and the closest continuous predicted disease segmentation, using the distance between site centroids. For the patient-by-patient comparison, we computed the metrics between the manual and predicted segmentations within the entire liver. For global DSC, all images were stacked together, this metric comes from the LiTs challenge15. All metrics were computed at the original image resolution.

The sensitivity of the model was evaluated on a site-by-site basis, where disease was considered identified if at least 45% of the ground truth overlapped with prediction. False-positive volume was defined as the volume of the predicted segmentation outside of the ground truth segmentation and was comprised of two errors: over-segmentation and erroneous segmentation. Erroneous false-positive volume was quantified as the volume that was completely unconnected from any ground truth segmentations and the over-segmentation false volume as the total predicted volume minus the erroneous volume (Figure 5).

## Qualitative Evaluation

For large structures, a high DSC can hide clinically important inaccuracies, while for smaller structures, a low DSC can be overly critical23. Our qualitative assessment of both CRLM and ablation segmentations was performed by a radiology-trained physician fellow (1, XXX) and two board-certified radiologists (2, XXX), an interventional radiologist, and (3, XXX), a diagnostic radiologist, both of whom have more than 10 years of experience.

We retrospectively identified 18 patients, who had previously undergone targeted thermal ablation therapy at our institution under an Institutional Review Board-approved study; these patients had 24 CRLM sites and 19 ablation zones. The predicted CRLM and ablation contours were scored independently based on a Likert quality scale of 1-5 (Supplementary Table 1).

# Results

The best validation loss scores were, Standard UNet: 0.041, Residual UNet: 0.024, DenseUNet: 0.016, DenseNet2D (Encoder frozen): 0.022, DenseNet2D (All trainable): 0.013, DenseNet3D (2D frozen): 0.011, DenseNet3D (All trainable): 0.0092.

## Quantitative

The best model was the Hybrid-WNet model: the 3D-UNet contained two layers, two convolution blocks, and 32 filters and had a convolution lambda of two. The model consisted of 14,497,600 parameters (14,408,960 trainable and 88,640 non-trainable).

A seed value of 0.67 and threshold value of 0.30 resulted in the highest overall DSC in the validation dataset. LiTs test set (n=70) predictions required a mean of 9.58 seconds, with a standard deviation of 2.32 seconds.

### Site-by-Site evaluation

#### 3D-IRCADb

On a site-by-site basis, the median surface distance, DSC, and sensitivity are presented in Table 3. Sites are distributed into two groups based on diameter: sites < 15 mm and ≥ 15 mm. For sites ≥ 15 mm (n=48), mean DSC was 0.74 and sensitivity was 98%. For sites < 15 mm (n=73), mean DSC was 0.16 and sensitivity was 23%.

#### Institutional Data

For the pre-treatment CECT CRLM target disease (n=19 sites), the mean (min-max) DSC was 0.80 (0.59-0.91), with 84% (16/19) having DSC ≥ 0.76. For the post-treatment CECT target ablation zones (n=14 sites), the mean (min-max) DSC was 0.75 (0.09-0.90), with 71% (10/14) having DSC ≥ 0.76.

### Patient-by-Patient evaluation

Patient-by-patient evaluation for 3D-IRCADb and institutional data are summarized in Table 4.

#### 3D-IRCADb

Quantitative metrics on a patient-by-patient basis for 3D-IRCADb are summarized in the top of Table 4. Median DSC was 0.74, median surface distance was 1.95 mm, and median false-positive discoveries per patient was 1. Note that a single patient might have multiple disease sites of varying sizes. The global DSC score was 0.81.

#### Institutional Data

Quantitative metrics for the institutional data are summarized in the middle and bottom of Table 4. For the pre-treatment CECT CRLM target disease (n=15 patients), the median DSC was 0.78, median surface distance was 0.78 mm, and false-positive discoveries per patient was 1. For the post-treatment CECT target ablation zones (n=9 patients), the median DSC was 0.79, median surface distance was 0.76 mm, and false-positive discoveries per patient was 2.

### Qualitative Evaluation

#### Institutional Data

Supplementary Table 2 shows the Likert scores of the two radiologists and radiology trained physician fellow for each targeted disease and ablation zone. The majority vote mean (min-max) Likert scores for the target disease volumes (n=24) were 4.8 (4-5). The majority vote mean (min-max) Likert scores for targeted ablation volume (n=19) were 4.1(2-5). All (24/24) of the CRLM contours and 84% (16/19) of the ablation contours had a majority Likert score ≥4. Figure 6 shows the scores of the two radiologists and physician fellow on the CRLM and ablation zones.

# Discussion

In this study, we proposed a Hybrid-WNet model architecture for the segmentation of the disease sites and ablation areas in the context of CRLM treated with ablation therapy. The model was further evaluated using the Likert scoring method by experts in the field of abdominal imaging and liver ablation with majority voting scores ≥4 out of 5 for 100%(24/24) of the tumor and 84%(16/19) of the ablation zone segmentations. The prediction process has been implemented in a commercial treatment planning system (RayStation 9B, RaySearch Laboratories, Sweden)24 and can perform segmentations in less than 30 seconds, making it highly suitable for clinical use. Our work is highly innovative because of the Hybrid-WNet model architecture and the training of the model; the model had 98% (47/48) sensitivity on disease sites ≥15mm, with predictions in <30 seconds.

While fully convolutional networks have been previously investigated for the segmentation of CRLM12, only a quantitative assessment of model performance has been reported. A comparison of our model results to those of other authors is given in Table 5; CRLM size distributions match those of previous work and were compared on both a site-by-site basis and patient-by-patient basis. While our model performed better for larger CRLM (≥15 mm) by DSC and sensitivity compared to Vorontsov et al12, it performed worse for CRLM <10 mm. Similar results have been reported by Christ et al;11 however, their work focused on primary liver disease. Using our institutional data as a proxy for clinical use, 84% (16/19) of the disease sites had a DSC>0.76, which has been reported as the inter-observer variability for CRLM segmentation25.

The proposed Hybrid-WNet architecture in this study differed from the model which inspired it,13 with additions to the 2D feature extractor and connecting all of the convolutional layers within the convolution blocks between the encoding and decoding of the 3D-DenseNet. Previously published studies proposed similar architectures that were focused only on dense connections within a single convolutional block26–28. The present implementation allowed the model to receive inputs from every convolution of the same image size across the entire network. This global passing of convolution layers across the network removed bottlenecks in each convolution block, something that is particularly important with dense connections where convolutions can be re-used. The proposed model will be publicly available (Github: redacted for anonymization).

Segmentation of CRLM has historically been difficult because of the relatively small size of lesions and the large search area. The extent of disease can vary from patient to patient, from a single lesion to multiple lesions. Simple class weighting of disease would lead to favoring training in patients with more disease sites. Our novel proposed method of splitting the liver into slabs that are centered specifically on disease sites ensures that the model learns using a more balanced representation of data. The training workflow was designed to allow the model to learn from batches that contains cubic images from multiple patients at once, enabling the creation of a more generalized model.

Majority voting showed poor Likert score (<4) for three ablation sites. The predictions for these sites can be seen in the Supplementary Figure 2, where disagreement about the boundary of the ablation zone and the time needed for correction resulted in a range of scores from the reviewers. Suboptimal image quality during porto-venous CT acquisition phase might have negatively impacted ablation zone boundary identification in such patients. Optimizing imaging protocols used for intra-procedural CT during ablation interventions might overcome this limitation. The measurement of false-positive volume seemed to be highly biased when there were small amounts of over-segmentation on large tumors; this was the rationale for the creation of the erroneous and over-segmentation false-positive volumes. To ensure transparency, we included all three and relied on qualitative assessment to add weight to the quality of the contours.

The main limitation of this study was the low sensitivity in disease sites <10 mm in diameter compared with 98% sensitivity for sites >15 mm. The low sensitivity in small sites may be caused in part by the test data used, where several 10-15-mm tumors were present on only a single CT slice. Presently, the model has a prediction cutoff, which determines the minimum confidence for the model to segment a voxel as disease or normal tissue. The current cutoff was selected to balance the sensitivity and false-positive ratio. By reducing the cutoff from what is reported here, a 74% sensitivity (31/42) can be achieved for the < 10 mm diameter sites, however this came at the expense of additional false-positive identifications of lesions. The goal of this work was for the model to be reproducible by the public, therefore it was trained only on publicly available data, with limited small sites for training.

The population of patients undergoing liver ablation at our institution is dominated by tumors > 10 mm, driving the need to optimize segmentation to this size.  Supporting this focus, a 10-year review of colorectal liver metastasis ablation patients in Italy found the mean disease size to be 2.2cm (range 0.8 – 4.0 cm)29. The model reported here is currently being used in a randomized Phase II clinical trial with great success.

# Conclusions

The presented novel Hybrid-WNet model provided fast and accurate segmentations of CRLM and ablation-zones on contrast-enhanced CT scans, with high concordance with physician reviews. This model might allow rapid tumor and ablation zone segmentations that might assist the interventional radiologist to evaluate ablation technical success intraprocedurally in an accurate and timely fashion.

# Acknowledgements

The authors would like to acknowledge the Editing Services, Research Medical Library at XXX, funding from the Society of Interventional Radiology Foundation Allied Scientist Grant, NIH (NCI R01CA221971, NCI R01CA235564), and the Helen Black Image Guided Fund.

# References

1. 	Meijerink MR, Puijk RS, van Tilborg AAJM, et al. Radiofrequency and Microwave Ablation Compared to Systemic Chemotherapy and to Partial Hepatectomy in the Treatment of Colorectal Liver Metastases: A Systematic Review and Meta-Analysis. Cardiovasc Intervent Radiol. 2018;41(8):1189-1204. doi:10.1007/s00270-018-1959-3

2. 	Shady W, Petre EN, Do KG, et al. Percutaneous Microwave versus Radiofrequency Ablation of Colorectal Liver Metastases: Ablation with Clear Margins (A0) Provides the Best Local Tumor Control. J Vasc Interv Radiol. 2018;29(2):268-275.e1. doi:10.1016/j.jvir.2017.08.021

3. 	Ruers T, Van Coevorden F, Punt CJA, et al. Local Treatment of Unresectable Colorectal Liver Metastases: Results of a Randomized Phase II Trial. JNCI J Natl Cancer Inst. 2017;109(9). doi:10.1093/jnci/djx015

4. 	Gillams AR, Lees WR. Radio-frequency ablation of colorectal liver metastases in 167 patients. Eur Radiol. 2004;14(12):2261-2267. doi:10.1007/s00330-004-2416-z

5. 	Sotirchos VS, Petrovic LM, Gönen M, et al. Colorectal Cancer Liver Metastases: Biopsy of the Ablation Zone and Margins Can Be Used to Predict Oncologic Outcome. Radiology. 2016;280(3):949-959. doi:10.1148/radiol.2016151005

6. 	Calandri M, Yamashita S, Gazzera C, et al. Ablation of colorectal liver metastasis: Interaction of ablation margins and RAS mutation profiling on local tumour progression-free survival. Eur Radiol. February 2018. doi:10.1007/s00330-017-5273-2

7. 	Shady W, Petre EN, Gonen M, et al. Percutaneous Radiofrequency Ablation of Colorectal Cancer Liver Metastases: Factors Affecting Outcomes—A 10-year Experience at a Single Center. Radiology. 2016;278(2):601-611. doi:10.1148/radiol.2015142489

8. 	Kaye EA, Cornelis FH, Petre EN, et al. Volumetric 3D assessment of ablation zones after thermal ablation of colorectal liver metastases to improve prediction of local tumor progression. Eur Radiol. 2019;29(5):2698-2705. doi:10.1007/s00330-018-5809-0

9. 	Gillams A, Goldberg N, Ahmed M, et al. Thermal ablation of colorectal liver metastases: a position paper by an international panel of ablation experts, the interventional oncology sans frontières meeting 2013. Eur Radiol. 2015;25(12):3438-3454. doi:10.1007/s00330-015-3779-z

10. 	Software-Aided Imaging (Morfeus) for Confirming Tumor Coverage With Ablation in Patients With Liver Tumors, the COVER-ALL Study - Full Text View - ClinicalTrials.gov. https://clinicaltrials.gov/ct2/show/NCT04083378. Accessed August 28, 2020.

11. 	Christ PF, Elshaer MEA, Ettlinger F, et al. Automatic liver and lesion segmentation in CT using cascaded fully convolutional neural networks and 3D conditional random fields. In: Lecture Notes in Computer Science (Including Subseries Lecture Notes in Artificial Intelligence and Lecture Notes in Bioinformatics). Vol 9901 LNCS. Springer Verlag; 2016:415-423. doi:10.1007/978-3-319-46723-8_48

12. 	Vorontsov E, Cerny M, Régnier P, et al. Deep Learning for Automated Segmentation of Liver Lesions at CT in Patients with Colorectal Cancer Liver Metastases. Radiol Artif Intell. 2019;1(2):180014. doi:10.1148/ryai.2019180014

13. 	Li X, Chen H, Qi X, Dou Q, Fu C-W, Heng P-A. H-DenseUNet: Hybrid Densely Connected UNet for Liver and Tumor Segmentation from CT Volumes. IEEE Trans Med Imaging. June 2018:1-1. doi:10.1109/TMI.2018.2845918

14. 	Seo H, Huang C, Bassenne M, Xiao R, Xing L. Modified U-Net (mU-Net) with Incorporation of Object-Dependent High Level Features for Improved Liver and Liver-Tumor Segmentation in CT Images. IEEE Trans Med Imaging. 2020;39(5):1316-1325. doi:10.1109/TMI.2019.2948320

15. 	Bilic P, Christ PF, Vorontsov E, et al. The Liver Tumor Segmentation Benchmark (LiTS).; 2019.

16. 	3D-IRCADb 01 | IRCAD France. https://www.ircad.fr/research/3d-ircadb-01/. Accessed September 29, 2020.

17. 	He K, Zhang X, Ren S, Sun J. Deep Residual Learning for Image Recognition. http://image-net.org/challenges/LSVRC/2015/. Accessed July 28, 2020.

18. 	Huang G, Liu Z, Van Der Maaten L, Weinberger KQ. Densely Connected Convolutional Networks. https://github.com/liuzhuang13/DenseNet. Accessed July 28, 2020.

19. 	Abadi M, Agarwal A, Barham P, et al. TensorFlow: Large-Scale Machine Learning on Heterogeneous Distributed Systems. https://static.googleusercontent.com/media/research.google.com/en//pubs/archive/45166.pdf. Accessed December 10, 2017.

20. 	NVIDIA Tesla V100 | NVIDIA. https://www.nvidia.com/en-gb/data-center/tesla-v100/. Accessed July 28, 2020.

21. 	Abadi M, Agarwal A, Barham P, et al. TensorFlow: Large-Scale Machine Learning on Heterogeneous Distributed Systems. www.tensorflow.org. Accessed July 28, 2020.

22. 	Smith LN. Cyclical Learning Rates for Training Neural Networks. www.cs.toronto.edu/. Accessed July 28, 2020.

23. 	Anderson BM, Lin EY, Cardenas CE, et al. Automated Contouring of Contrast and Noncontrast Computed Tomography Liver Images With Fully Convolutional Networks. Advancesradonc. 2020. doi:10.1016/j.adro.2020.04.023

24. 	Bodensteiner D. RayStation: External beam treatment planning system. Med Dosim. 2018;43(2):168-176. doi:10.1016/j.meddos.2018.02.013

25. 	Jensen NKG, Mulder D, Lock M, et al. Dynamic contrast enhanced CT aiding gross tumor volume delineation of liver tumors: An interobserver variability study. Radiother Oncol. 2014;111(1):153-157. doi:10.1016/j.radonc.2014.01.026

26. 	Jégou S, Drozdzal M, Vazquez D, Romero A, Bengio Y. The One Hundred Layers Tiramisu: Fully Convolutional DenseNets for Semantic Segmentation. https://github.com/SimJeg/FC-DenseNet. Accessed August 26, 2020.

27. 	Hai J, Qiao K, Chen J, et al. Fully convolutional densenet with multiscale context for automated breast tumor segmentation. J Healthc Eng. 2019;2019. doi:10.1155/2019/8415485

28. 	Brahimi S, Ben Aoun N, Benoit A, Lambert P, Ben Amar C. Semantic segmentation using reinforced fully convolutional densenet with multiscale kernel. Multimed Tools Appl. 2019;78(15):22077-22098. doi:10.1007/s11042-019-7430-x

29. 	Solbiati L, Ahmed M, Cova L, Ierace T, Brioschi M, Goldberg SN. Small liver colorectal metastases treated with percutaneous radiofrequency ablation: Local response rate and long-term survival with up to 10-year follow-up. Radiology. 2012;265(3):958-968. doi:10.1148/radiol.12111851

# Table and Figure Legends

Table 1: Image Acquisition Parameters of the LiTS Challenge for the Training, Validation, and Test Sets

Table 2: Investigated architectural hyper parameters for each architectural style. Note that the number of filters doubled after each pooling. ‘Growth Rate’ is unique to the Dense network.

Table 3: Mean, Min, and Max Dice Similarity Coefficient and Median Surface Distance and Sensitivity for Individual Disease Sites by Differing Size Criteria of the 3D-IRCADb Dataset

Table 4: Metrics of Dice Similarity Coefficient, Median Surface Distance (mm), False-Positive Discoveries (Per Patient), and False-Positive Volume (cc) for 15 3D-IRCADb Test Patients

Table 5: Comparison of CRLM Segmentation Results from Our Model and the Literature. Green Box Indicates More Disease Sites (n), Better Dice Similarity Coefficient (DSC), or Higher Sensitivity.

Supplementary Table 1: Likert Scoring for Assessment of Colorectal Liver Metastasis (CRLM) and Ablation Contours

Supplementary Table 2: Breakdown of Likert Score (1-5) for Predicted Contours of CLM and Ablation Sites by Each Reviewer

Figure 1: Liver distributed into individual slabs of 32x120x120. Disease was labeled as disease, regardless of the center of the slab. The validation and test set were not broken into slabs; our architectures accepted variable input sizes, with the entire liver being passed at once for evaluation and testing.

Figure 2: Top (A): Basic architecture framework. Bottom (B): Difference in convolution blocks, surrounded by green to indicate the same region in (A). Standard: previous feature maps are convolved and activated. Residual: previous feature maps are directly added to convolutional output in a skip-connection before activation. Dense: previous feature maps are continually concatenated together before activation and convolution. BN: batch normalization.

Figure 3: Hybrid-WNet. (Left) Pre-trained 2D DenseNet 121 converted into UNet, where final features were concatenated into 3D DenseNet. (Right) 3D DenseNet architecture was defined as two layers, two convolution blocks in layer 0, three convolution blocks in layer 1, and eight initial filters. BN: Batch Normalization

Figure 4: Example of visualization of prediction on validation data during training. Top row: Image being fed into the model, masked by the liver segmentation. Middle row: Ground truth of disease contours. Bottom row: Prediction of disease contours, set with a threshold of 0.5 to the binary mask.

Figure 5: (Left) Overlay between predicted (white) and ground truth (red) disease segmentations. (Right) Subtraction of the predicted and ground truth disease segmentation and over-segmentation of the disease volume (blue) and unconnected erroneous false-positive region (green). The sum of blue and green is a false-positive volume.

Figure 6: (Top) Likert score by each reviewer for each disease site. (Bottom) Likert score by each reviewer for each ablation site. A higher value is associated with higher quality.

Supplementary Figure 1: Learning rate finder figure. The learning rate is gradually increased from 10e-7 to 10e-0; the point where loss begins to decrease is the minimum learning rate (min LR), and the point where it begins to increase is the maximum learning rate (max LR).

Supplementary Figure 2: Three cases where majority voting had a Likert Score less than 4. (Top) We believe that the top failed due to similarity in appearance to the gallbladder. (Middle, Bottom) Model over-segmented regions around the ablation zones.