Improving liver ablation treatment for colorectal liver metastases: retrospective assessment of minimum margin to for image guidance and treatment assessment

Brian M. Anderson1,2, Yuan-Mao Lin3, Ethan Y. Lin3, Guillaume Cazoulat1, Sanjay Gupta3, *Bruno C. Odisio3, *Kristy K. Brock1

1) Department of Imaging Physics, The University of Texas MD Anderson Cancer Center, Houston, Texas 77030

2) The University of Texas Graduate School of Biomedical Sciences at Houston, Houston, Texas 77225

3) Department of Interventional Radiology, The University of Texas MD Anderson Cancer Center, Houston, Texas 77225

*Co-senior authors

Preliminary results from this work were presented at the annual SPIE conference in 2018, but did not include our work with recurrence location assessment.

Acknowledgements: The authors would like to acknowledge the scientific editors in the Research Medical Library at The University of Texas MD Anderson Cancer Center and funding from the Society of Interventional Radiology Foundation Allied Scientist Grant, the NIH (NCI R01CA235564), and the Helen Black Image Guided Fund.

Keywords: Colorectal liver metastases, colorectal cancer, liver ablation therapy, biomechanical model-based deformable image registration

# INTRODUCTION

Colorectal cancer (CRC) is a leading cause of cancer-related death worldwide1, and is the third most common cancer in the United States2. 25% of CRC patients present with colorectal liver metastasis (CLM) at the time of diagnosis, and up to 60% will develop CLM  over their disease course3.  About two-thirds of deaths resulting from CRC are believed to be caused by CLM4. While curative hepatic resection is the treatment of choice for patients presenting with CLM5, only 20% of patients are candidates for surgery.  For the remaining patients, systemic treatment or other forms of loco-regional therapy such thermal liver ablation are offered6–9. Liver ablation has demonstrated 5-year overall survival rates of 24% to 51% 10–12, and significantly longer overall survival when combined with systemic chemotherapy compared with chemotherapy alone13.

Minimal radiographic ablation margins has been described as one of the most relevant factors associated with improved local tumor control following thermal liver ablation14–1718, with minimum margins of 5 -10 mm suggested to achieve optimal local control 19,20. Despite its criticality, ensuring sufficient ablation margin can be challenging for several reasons. First, tumor visualization is limited on the intraprocedural non–contrast-enhanced computed tomography (CT) or ultrasound images for tumor targeting and probe guidance. Second, it is nearly impossible to distinguish between disease and ablated tissue in the post-treatment contrast-enhanced CT scan for hypo-enhancing disease, preventing accurate measurement of delivered margin. Finally, accurate 3-dimensional (3D) assessment of ablation margin is difficult as rigid registration is insufficient due to liver deformation throughout the procedure and to the inherent inability to effectively assess ablation zone on a constrained intra-procedural time-period without the aid of high-level computational analysis.

Current methods of ablation assessment often rely on 2D distance measurement to a visible landmarks on the post-treatment imaging, and do not ensure 3D minimum margin. A workflow has been recently published describing the measurement of a 3D margin; however, the work relies on rigid registration that terminates when target registration errors greater than 3 mm are present21, which commonly occur given liver deformations associated with patient’s breathing and changes in position22, and post-ablation volumetric changes.

We hypothesized that the minimum ablation margin, calculated following the use of a biomechanical model-based deformable image registration (DIR), will differentiate patients with local tumor progression and those receiving sufficient ablation delivery.

# MATERIALS AND METHODS

Data from 30 patients whose liver metastasis from colorectal cancer had been treated with local PTA at XXX were retrospectively evaluated under a prospectively-maintained liver ablation registry approved by the institutional review board. Patients were included if they had a contrast-enhanced pretreatment CT image within 45 days of PTA procedure, post-treatment contrast-enhanced porto-venous phase immediately performed at the end of PTA procedure. The cohort comprised 14 patients with post-ablation local tumor progression (LTP) and 16 without LTP. Patients’ characteristics are listed in Table 1.

### Imaging data

Tri-phasic (arterial, portal-venous, washout) CT images (minimum slice thickness, 2 mm; maximum slice thickness, 5 mm; minimum image resolution, 0.5625 mm/pixel; maximum image resolution, 0.8789 mm/pixel) were obtained with iodinated intravenous contrast injection prior and after to the ablation procedure for tumor identification and ablation zone assessment, respectively.

All imaging data were uploaded into the FDA-approved radiation therapy treatment planning system (RayStation v5.0.2, RaySearch Laboratories, Stockholm, Sweden). The liver was manually segmented on all portal venous pretreatment, intraprocedural guidance, and post-treatment CT images. The venous phase of the pretreatment tri-phasic CT was used to delineate the gross tumor volume (GTV) region of interest (ROI), as the contrast between normal tissue and disease was greatest in this phase. The post-treatment venous-phase CT image was used to define the ablation zone. Segmentations of the GTV, ablation zone, and recurrent disease were performed by a graduate student under the guidance of a radiology-trained physician fellow and approved by a board-certified Diagnostic and interventional radiologist with 11 years of experience in interventional oncology (Figure 1, Figure 2).

Biomechanical Modeling

The biomechanical-model based DIR Morfeus, which is integrated with the RayStation system, was employed for this work23,24. Pre-treatment and post-treatment CTs are first rigidly registered using an automated gray-level cross correlation of the liver volume ROIs. Morfeus creates of a triangular surface mesh, which defines the relationship between pre-treatment and post-treatment image sets. The triangular mesh was assigned a Young’s modulus of 1000 pascals and Poisson’s ratio of 0.45, values which represent the biomechanical properties of the liver, stiffness and compressibility, respectively, in a linear elastic material model. The correspondence of the generated triangular mesh determined the boundary conditions for the finite element analysis. Morfeus has been shown to have an accuracy on the order of the image voxel size when registering liver images (CT to CT and CT to magnetic resonance  imaging) for image-guided radiation therapy applications 25,26.

We investigated permuting two parameters, the smoothing radius and triangular-mesh edge length, in the creation of the triangular mesh. We investigated triangular mesh edge values of 3, 6, 9, and 12 mm (default 6 mm) and smoothing radii of 1, 3, and 5 mm (default 1 mm). The metric of success for our selection of these parameters was the Dice similarity coefficient27 between the generated triangular mesh and the manually defined slice-based contours on a patient-by-patient basis.  The finite element analysis resulted in a dense and symmetric deformation vector field, which was resampled to the ROI of the liver contour. The ablation zone was then propagated from the post- to pre-treatment image set using the deformation vector field.

## Assessing Ablation

With the ablation-zone propagated onto the pre-treatment imaging, we computed a 3D minimum distance to agreement (DTA) between the GTV and ablation zone following rigid registration alone and rigid registration combined with biomechanical registration (Figure 3). Regions in which the ablation zone margin entirely encompassed the GTV were considered to have positive ablation margins, and regions where the GTV and ablation region overlapped had a margin of 0mm. In rigid registration, regions where the disease expanded beyond the ablation zone were considered to have a margin of 0mm. To remove bias of minimum margin assessment when the GTV was on a subcapsular location, the ablation zone was expanded by 10 mm everywhere outside of the liver. This ensured that the minimum margin is accurately recorded as the minimum margin within the liver (add a figure because this will be confusing xxx).

The use of deformable registration to distinguish locally-recurring from non-locally-recurring patients was assessed using a Mann-Whitney U-Test28, as the data is not paired and not continuous since the minimum DTA has a close limit of 0 mm.

## Assessing Recurrence

To quantify the correlation of 3D minimum margin with the location of tumor progression, we had several challenges to overcome. One of the most obvious challenges is physical change in ablation cavity size, our data showed an average decrease of 37% in ablation cavity volume from post-treatment to recurrence images, likely due to dehydration of the tissue after thermal ablation and autophagy of the treated tissue. This volumetric change in the ablation site helps illustrate why simple use of whole-liver rigid registration is not sufficient to accurately align recurrence sites with minimum ablation margins. A rigid registration, instead of a deformable registration, was performed to avoid the potential introduction of unknown errors, as there is no validated deformable registration available to account for ablation zone shrinkage.

We developed a workflow involving the registration of the ablation and follow-up CT about the liver vasculature, and developed a ray-tracing algorithm to assess correspondence of the recurrence to the 3D minimum ablation margin. Since the registration is driven by the vessels, we are confident that areas of large asymmetry in the recovering ablation zone with proximity to vessels will still be oriented correctly. We do not want to register based on the vessels in the entire liver, however, as this will simply turn into a whole-liver registration. Instead, we wish to focus on the vasculature local to the ablation zone.

Liver vasculature was automatically segmented using an in-house vesselness algorithm on the post-treatment CT and the recurrence CT29,30. A sphere of interest with a radius of 5cm, 7.5cm, and 10cm was defined on both images, centered on the ablation zone in the post-treatment CT and the recurrence CT. The 7.5-cm radius was chosen on the basis of visual inspection of the completed registration for all patients.

A rigid registration, instead of a deformable registration, was performed to avoid the potential introduction of unknown errors, as there is no validated deformable registration available to account for ablation zone shrinkage. If sufficient vasculature could not be identified, manual registration was performed to best align the ablation zones on the post-treatment and recurrence images under the guidance of a trained interventional radiologist. To reduce bias, the radiologist was unaware of where the predicted minimum margin was located.

To create a quantitative metric of the relationship between the identified minimum margin on the post-treatment CT and the recurrence identified on the follow-up CT, we developed a 3D ray-trace based on spherical coordinates to determine if the recurrence and the minimum ablation margin correlate to the same spherical location.  Spherical coordinate’s comparison requires that the ablation zone in both post-treatment and recurrence imaging be correctly registered translationally and rotationally. While we assume asymmetric shrinkage of the ablation zone in areas proximal to large vessels, our registration is driven by similarities in the vessels, which should be robust to deviations in ablation zone recovery.

Figure 4 illustrates the ray-trace process, in which a 3D cone was created radiating from the center of the ablation zone on the recurrence CT, and the cone intersected every point of the contoured recurrence. For visualization purposes, this cone was then mapped onto the centroid of the ablation zone on the post-treatment image. If the mapped cone overlapped with the minimum margin, the recurrence was considered to have occurred in the same relative region that the minimum margin existed. If the mapped cone did not overlap with the identified margin, the recurrence was considered to be unrelated to the minimum margin.

The use of deformable registration to distinguish locally-recurring from non-locally-recurring patients was assessed using a Mann-Whitney U-Test28, as the data is not paired and not continuous since the minimum DTA has a close limit of 0 mm.

RESULTS

Rigid and deformable registration between the pretreatment images and both the intraprocedural and post-treatment images was successfully completed for all 30 patients.

For the deformable registration, the mean (min-max, median) minimum ablation margin around the GTV for patients without local recurrence was 3.19 mm (0.70-6.10, 2.90 mm), compared to patients diagnosed with local recurrence 1.14 mm (0-5.6, 0). Only 3 patients in the local recurrence group had margins greater than 2 mm. A Mann-Whitney U test revealed a significant (P < 0.01) difference between the 2 groups based on 3D minimum distance to agreement (Table 2).

Rigid registration resulted in significant errors in the mapping of disease, where mapped disease was shown to exist in normal liver tissue. Reminder, the disease will appear hypo-enhancing on portal venous phase image in the post-treatment, similarly to the pre-treatment. For this reason we would expect little to no amount of GTV volume present outside of the ablation zone after mapping.

The mean (std) of the percentage GTV volume mapped outside of the ablation zone for rigid registration was 19.51% (28.26%), while for deformable registration it was 1.39% (3.80%) (Table 2).

## Assessing Recurrence

In 2 cases, the contrast enhancement was insufficient to identify nearby vasculature, so manual rigid registration was performed focused on the ablation area. Of the 14 patients who experienced a recurrence, 12 (86%) had an overlap between the recurrence and the 5-mm expansion of disease outside of the ablation zone, indicating that the lack of 5 mm of margin correlated with where future progression occurred. The median volume overlap between the projected cone and the minimum margin was 0.34 cc (range, 0-3.38 cc) (Figure 6). Despite the fact that the volume of overlap can be very small, the consensus among our reviewing physicians is that the location of 5-mm expansion coincides with actual recurrence (Figure 7).

# DISCUSSION

The efficacy of the proposed workflow was shown by distinguishing between the retrospectively obtained patients who experienced local recurrence from those who did not with high statistical significance using a Mann-Whitney U test (P < 0.01). We observed overlap between future recurrence and the minimum ablation margin in 86% of patients, and we believe this indicates that the minimum ablation margin is informative of where recurrence might develop. Our study demonstrated that the mean difference in minimum ablation margin between the group of patients who experienced local recurrence and the group who did not was particularly large (2 mm, nearly 50% of the desired minimum margin for microwave therapy) and statistically significant (P < 0.01).

Rigid registration alone is not sufficient to properly identify minimum ablation margin. Rigid registration led to multiple cases of the disease mapped outside of the ablation zone entirely  (Panel A, Figure 8). Most notably, patient 2 in the local recurrence group was recorded as having a minimum DTA of 5.3mm via rigid registration, offering a false sense of confidence compared to the deformed registration (Panels B and C, Figure 8).

Our proposed workflow has the potential to improve local control rates in patients, by assessing the minimum DTA immediately following ablation, if this can be performed in a timely manner that does not adversely affect the clinical workflow. The development of deep learning for liver segmentation has removed the previous challenges in clinical implementation due to manual segmentation31.  The process is fully automated after receiving segmentations of the GTV, ablation zone, and liver, with run times of approximately 5 minutes. The result returns the ablation margin on the pretreatment image and an image showing the distribution of ablation zone. This distribution can assist the physician in identifying where further ablation may be desired.

In addition to determining the ablation margins, the algorithms proposed in this research can also be employed to ensure accurate placement of the probe into the tumor, especially in cases where contrast is no longer present in the image of the probe artifact masks the tumor visibility. This would require a CT scan of the entire liver once the probe is inserted to perform biomechanical model-based DIR to map the tumor onto the image with the probe to evaluate accuracy of placement.

Our work is primarily limited by the paucity of patients presenting with CLM who also had contrast-enhanced CT scans performed up to 45 days before treatment and post-treatment scans on the day of treatment. The amount of volume overlap between recurrence and 5-mm minimum margin also was small, less than 0.5 cc for 75% (8/12) of the patients in whom overlap was present. However, it is important to note that we would expect that the overall volume of the 5-mm margin would already be very small, and therefore any overlap should also be small.

Our work shows that the use of a biomechanical model-based DIR can help determine if sufficient ablation margin has been delivered and correlate the location of the recurrence with the minimum ablation area. These results show the potential for clinical benefit.

# REFERENCES

1. 	WHO Biennial Report 2012-2013. Accessed October 27, 2020. https://governance.iarc.fr/SC/SC50/Biennial Report 2012-2013.pdf

2. 	Siegel RL, Miller KD, Jemal A. Cancer statistics, 2019. CA Cancer J Clin. 2019;69(1):7-34. doi:10.3322/caac.21551

3. 	Alper Sag A, Selcukbiricik F, Mandel NM. Evidence-based medical oncology and interventional radiology paradigms for liver-dominant colorectal cancer metastases. World J Gastroenterol. 2016;22(11):3127-3149. doi:10.3748/wjg.v22.i11.3127

4. 	Tomlinson JS, Jarnagin WR, DeMatteo RP, et al. Actual 10-year survival after resection of colorectal liver metastases defines cure. J Clin Oncol. 2007;25(29):4575-4580. doi:10.1200/JCO.2007.11.0833

5. 	Abdalla EK, Vauthey J-N, Ellis LM, et al. Recurrence and outcomes following hepatic resection, radiofrequency ablation, and combined resection/ablation for colorectal liver metastases. Ann Surg. 2004;239(6):818-825; discussion 825-7. doi:10.1097/01.SLA.0000128305.90650.71

6. 	Liu L-X, Zhang W-H, Jiang H-C. Current treatment for liver metastases from colorectal cancer. World J Gastroenterol. 2003;9(2):193. doi:10.3748/WJG.V9.I2.193

7. 	Shady W, Petre EN, Gonen M, et al. Percutaneous radiofrequency ablation of colorectal cancer liver metastases: Factors affecting outcomes-a 10-year experience at a single center. Radiology. 2016;278(2):601-611. doi:10.1148/radiol.2015142489

8. 	Ahrq. Local Hepatic Therapies for Metastases to the Liver From Unresectable Colorectal Cancer Comparative Effectiveness Review Number 93. Accessed October 18, 2019. www.effectivehealthcare.ahrq.gov/reports/final.cfm.

9. 	Weng M, Zhang Y, Zhou D, et al. Radiofrequency Ablation versus Resection for Colorectal Cancer Liver Metastases: A Meta-Analysis. Cheng JQ, ed. PLoS One. 2012;7(9):e45493. doi:10.1371/journal.pone.0045493

10. 	Gillams AR, Lees WR. Five-year survival in 309 patients with colorectal liver metastases treated with radiofrequency ablation. Eur Radiol. 2009;19:1206-1213. doi:10.1007/s00330-008-1258-5

11. 	Kim KH, Yoon YS, Yu CS, et al. Comparative analysis of radiofrequency ablation and surgical resection for colorectal liver metastases. J Korean Surg Soc. 2011;81(1):25. doi:10.4174/jkss.2011.81.1.25

12. 	Solbiati L, Ahmed M, Cova L, Ierace T, Brioschi M, Goldberg SN. small liver colorectal Metastases Treated with Percutaneous radiofrequency ablation: Local Response Rate and Long-term Survival with Up to 10-year Follow-up 1. Radiol n Radiol. 2012;265. Accessed December 18, 2017. http://pubs.rsna.org/doi/pdf/10.1148/radiol.12111851

13. 	Ruers T, Van Coevorden F, Punt CJA, et al. Local Treatment of Unresectable Colorectal Liver Metastases: Results of a Randomized Phase II Trial. JNCI J Natl Cancer Inst. 2017;109(9). doi:10.1093/jnci/djx015

14. 	Berber E, Pelley R, Siperstein AE. Predictors of Survival After Radiofrequency Thermal Ablation of Colorectal Cancer Metastases to the Liver: A Prospective Study. J Clin Oncol. 2005;23(7):1358-1364. doi:10.1200/JCO.2005.12.039

15. 	Abitabile P, Hartl U, Lange J, Maurer CA. Radiofrequency ablation permits an effective treatment for colorectal liver metastasis. Published online 2006. doi:10.1016/j.ejso.2006.10.040

16. 	Hamada A, Yamakado K, Nakatsuka A, et al. Radiofrequency ablation for colorectal liver metastases: prognostic factors in non-surgical candidates. Jpn J Radiol. 2012;30(7):567-574. doi:10.1007/s11604-012-0089-0

17. 	Liang P, Dong B, Yu X, et al. Prognostic Factors for Percutaneous Microwave Coagulation Therapy of Hepatic Metastases. Am J Roentgenol. 2003;181(5):1319-1325. doi:10.2214/ajr.181.5.1811319

18. 	Kim Y, Lee WJ, Rhim H, Lim HK, Choi D, Lee JY. The Minimal Ablative Margin of Radiofrequency Ablation of Hepatocellular Carcinoma (&gt; 2 and &lt; 5 cm) Needed to Prevent Local Tumor Progression: 3D Quantitative Assessment Using CT Image Fusion. Am J Roentgenol. 2010;195(3):758-765. doi:10.2214/AJR.09.2954

19. 	Sotirchos VS, Petrovic LM, Gönen M, et al. Colorectal Cancer Liver Metastases: Biopsy of the Ablation Zone and Margins Can Be Used to Predict Oncologic Outcome. Radiology. 2016;280(3):949-959. doi:10.1148/radiol.2016151005

20. 	Odisio BC, Yamashita S, Huang SY, et al. Local tumour progression after percutaneous ablation of colorectal liver metastases according to RAS mutation status. Br J Surg. 2017;104(6):760-768. doi:10.1002/bjs.10490

21. 	Kaye EA, Cornelis FH, Petre EN, et al. Volumetric 3D assessment of ablation zones after thermal ablation of colorectal liver metastases to improve prediction of local tumor progression. doi:10.1007/s00330-018-5809-0

22. 	Heizmann O, Zidowitz S, Bourquain H, et al. Assessment of intraoperative liver deformation during hepatic resection: Prospective clinical study. World J Surg. 2010;34(8):1887-1893. doi:10.1007/s00268-010-0561-x

23. 	Brock KK, Sharpe MB, Dawson LA, Kim SM, Jaffray DA. Accuracy of finite element model-based multi-organ deformable image registration. Med Phys. 2005;32(6Part1):1647-1659. doi:10.1118/1.1915012

24. 	Velec M, Moseley JL, Svensson S, Hårdemark B, Jaffray DA, Brock KK. Validation of biomechanical deformable image registration in the abdomen, thorax, and pelvis in a commercial radiotherapy treatment planning system. Med Phys. 2017;44(7):3407-3417. doi:10.1002/mp.12307

25. 	Brock KK, Dawson LA, Sharpe MB, Moseley DJ, Jaffray DA. Feasibility of a novel deformable image registration technique to facilitate classification, targeting, and monitoring of tumor and normal tissue. Int J Radiat Oncol Biol Phys. 2006;64(4):1245-1254. doi:10.1016/j.ijrobp.2005.10.027

26. 	Voroney J-P, Brock KK, Eccles C, Haider M, Dawson LA. Prospective comparison of computed tomography and magnetic resonance imaging for liver cancer delineation using deformable image registration. Int J Radiat Oncol. 2006;66(3):780-791. doi:10.1016/J.IJROBP.2006.05.035

27. 	Dice LR. MEASURES OF THE AMOUNT OF ECOLOGIC ASSOCIATION BETWEEN SPECIES. Accessed November 6, 2019. https://about.jstor.org/terms

28. 	McKnight PE, Najab J. Mann-Whitney U Test. In: The Corsini Encyclopedia of Psychology. John Wiley & Sons, Inc.; 2010:1-1. doi:10.1002/9780470479216.corpsy0524

29. 	Cazoulat G, Elganainy D, Anderson BM, et al. Vasculature-Driven Biomechanical Deformable Image Registration of Longitudinal Liver Cholangiocarcinoma Computed Tomographic Scans. Adv Radiat Oncol. 2020;5(2):269-278. doi:10.1016/j.adro.2019.10.002

30. 	Cazoulat G, Owen D, Matuszak MM, Balter JM, Brock KK. Biomechanical deformable image registration of longitudinal lung CT images using vessel information. Phys Med Biol. 2016;61(13):4826-4839. doi:10.1088/0031-9155/61/13/4826

31. 	Anderson BM, Lin EY, Cardenas CE, et al. Automated Contouring of Contrast and Noncontrast Computed Tomography Liver Images With Fully Convolutional Networks. Adv Radiat Oncol. 2020;0(0). doi:10.1016/j.adro.2020.04.023